Setting the LN03 printer for different page sizes.

B. Z. Lederman

2572 E. 22nd St.

Brooklyn, N.Y. 11235-2504

The  LN03  printer  can  print  text  in  different  sizes  and   different

orientations.   The normal (default) setting is called "portrait" mode (the long

side of the paper is vertical, as in a normal letter), and 10 pitch  (characters

per  inch,  which  is  about the same as "pica" typewriting) which allows for 80

columns across the page.  It is also possible to print 12 pitch (about the  same

as  "elite" typewriting) and 13.6 pitch to have more columns on the page, and to

print in "landscape" mode (the long side of the paper is horizontal) to put more

text on a page.

NOTE

Columns means the number of characters which will  print  across

the  page:   it  does  not have the same meaning as columns in a

spread sheet, which may be many characters in width.

There are no switches or controls on the LN03  to  obtain  these  settings:

they  must  be  sent  to  the  printer  from the computer.  Important:  once the

printer has been set to a particular format, it will remain in that format until

one  of  the  following  occurs:  the printer is given a new format command; the

printer is given a reset command which will put it back to the default settings;

or  the printer power is turned off.  Therefore, it is necessary that anyone who

wishes to print a non-standard format coordinate with the other  users  of  that

printer so that others do not unexpectedly receive the wrong format.  You should

also reset the printer to the default settings when you have finished.

To  set  the  printer  characteristics,  a  new  form  has  been  added  to

ALL-IN-ONE.   This  form is not yet on any menu, but may be accessed directly by

entering the command PRINTER from any major form (such  as  Electronic  Mail  or

Word Processing).

Set Printer Characteristics

RESET  Reset printer

P80    Portrait mode, 80 columns (normal default value)

P96    Portrait mode, 96 columns (elite or 12 pitch)

P108   Portrait mode, 108 columns, 85 lines

L100   Landscape mode, 100 columns

L120   Landscape mode, 120 columns

L132   Landscape mode, 132 columns

10L    Mailing Labels, 10 lines high

19L    Mailing Labels, 19 lines high

Enter selection and press RETURN

Setting the LN03 printer for different page sizes.                        Page 2

B. Z. Lederman                                                     28 March 1986

After entering one of the selections, you will be prompted for the  printer  you

wish  to use, just as you would when printing any other document.  The following

menu will appear in the lower part of your screen:

Printing Document

Document Destination:

Printer Type:

Number of Copies:     1

Enter information and press RETURN or

Press GOLD A to change standard print settings

You may specify the printer you want to set:  it must be  the  same  printer  on

which  you  will  print  the  document.   If  you  don't  remember  the printers

available, enter blank spaces in the destination field and  press  GOLD L.   You

should  not use GOLD A here as the page settings have no meaning for the printer

characteristics files.

When you set the page characteristics for one of these  non-standard  print

settings, you will need to know how many lines will print on the page.

Format   Width | Printable Lines   Total Lines per page

|

RESET          |  reset to default (same as PORTRAIT 80)

|

PORTRAIT   80  |  63                       66

PORTRAIT   96  |  63                       66

PORTRAIT  108  |  87                       88  (at 8 LPI)

|

LANDSCAPE 100  |   *

LANDSCAPE 120  |   *

LANDSCAPE 132  |  64                       68

|

10L            |                           10

19L            |                           19

|

Printable lines are the number of lines which may contain text,  and  should  be

used when counting the lines that may be printed on a page.  The total lines per

page is the effective size of the paper, including the lines on which  no  print

will  appear.   The  two  formats  marked with an asterisk (*) have not yet been

finalized:  they could have either 47 or 66 lines,  depending  on  the  vertical

spacing setting which may be 6 or 8.  Landscape mode 132 columns is intended for

a vertical spacing of 8 LPI, and you should set your page layout for this value:

if you don't, WPS will set it back to 6 LPI and you will get extra pages as your

document run onto the next page.

Once the printer has been set to the desired mode, ALL documents  (such  as

Word  Processing  documents,  Spreadsheets,  or  Electronic  Mail)  sent to that

printer will print in that mode until the print  mode  is  changed.   The  RESET

command will send a sequence to reset the printer to it's default condition, and

should be sent after you are finished printing is a special mode.  This  command

may  cause  a  blank  sheet of paper to be output from the printer, or a copy of

your ALL-IN-ONE screen may appear

Setting the LN03 printer for different page sizes.                        Page 3

B. Z. Lederman                                                     28 March 1986

The formats which will be of greatest interest  are  the  default  setting  (the

setting in which most printing now occurs) which is the same as PORTRAIT mode 80

columns; PORTRAIT mode 96 columns, which is the same as 12 pitch or elite  type;

and  LANDSCAPE mode 132 columns which puts as many columns and lines on an 8" x

11" page as would be obtained on 11" x 14" paper at 10 pitch,  and  can  replace

printing on the large size computer paper.

The two settings 10L and 19L are for mailing labels:  10L is for the  blank

white labels, and 19L is for labels with a return address at the top.

When counting the number of lines per page, you must consider the number of

lines  printed  per inch.  WPS-Plus Word Processing allows you to set the number

of vertical characters (lines) per inch in the PS menu.  This  setting  WILL  be

sent  to  the printer, so if you want 87 lines in PORTRAIT.108 mode, or 67 lines

in LANDSCAPE.132 mode, you must be sure you have set  your  print  option  to  8

lines per inch.

An alternative method of changing mode is to include  the  commands  within

your own file or document:  this could be done within a word processing document

to change print characteristics for that document (or even for a portion of that

document).   At  the  present  time,  WPS-Plus  will  not process these commands

properly, but instructions for their use will be given here for future reference

should WPS-Plus be modified in the future.

A set of files has been created which  contain  the  commands  to  set  the

printer  to several common formats:  these have been placed in the system common

library so that anyone may use them.  The files have the following names:

Directory   Format  .Width

SYS$LIBRARY:RESET.PRINTER

SYS$LIBRARY:PORTRAIT.80

SYS$LIBRARY:PORTRAIT.96

SYS$LIBRARY:PORTRAIT.108

SYS$LIBRARY:LANDSCAPE.100

SYS$LIBRARY:LANDSCAPE.120

SYS$LIBRARY:LANDSCAPE.132

SYS$LIBRARY:10LINE.LA100

SYS$LIBRARY:19LINE.LA100

The directory SYS$LIBRARY is a symbolic name which always points to  the  system

common   library.   The  file  name  of  PORTRAIT  or  LANDSCAPE  indicates  the

orientation the text will have when printed, and the extension is the number  of

columns  (positions  or  characters)  which may be printed across the page.  The

most popular formats will be PORTRAIT.80 which corresponds to the default format

in  which  most  printing  is  now  done, PORTRAIT.96 which is similar but is 12

pitch, and LANDSCAPE.132, which will be useful for such items  as  spreadsheets.

Special  attention should be given to the RESET.PRINTER file:  this file sends a

reset sequence to put the printer back to it's default settings.  When  printing

text,  these  command  files  could  be included within your document to set the

format for that  document.   You  would  put  the  appropriate  command  at  the

beginning  of  the document (using the GET option to include a VMS file with one

of the names above, and reset  the  printer  at  the  end  of  the  document  by

including the RESET.PRINTER file.