ANALYSIS OF A SYSTEM CRASH

David Deley

Here's an example of how to examine a system crash dump file using the

Symbolic Dump Analyzer.  This may come in handy if you're trying to

debug some kernel mode code.  This particular crash happened while I was

developing the ZEROACC program.  It was interesting because the first

thing the program did in kernel mode was establish a condition handler,

yet the computer still crashed.

The following commands were used to obtain the information about the

system crash:

$ ANALYZE/CRASH SYS$SYSTEM:SYSDUMP.DMP

SDA> SHOW CRASH

SDA> SHOW STACK

SDA> SHOW STACK/USER

The annotated output from SDA is shown at the end of this file after the

discussion.  Note that the stack builds from the bottom up, so you'll

find #1, the starting point, near the bottom of the user stack shown at

the end of this file.  From that point you can read upwards to follow

the course of the program.

#1.

At #1. on the user stack the program starts.  The frame below #1. shows

the PC was originally in system space at EXE$IMGSTA+02F (image start)

when it called my program.

#2.

The program then calls SYS$CMKRNL.  It pushes an argument list on the

stack at #2a.  It saves register R4 as specified by the save register

mask at the beginning of routine SYS$CMKRNL at #2b.  And then it builds

the call frame itself up to the top of the stack.

#2c.

Routine SYS$CMKRNL executes the CHMK instruction.  The CHMK instruction

switches the stack pointer to point to the kernel stack, changes the

Processor Status Longword (PSL) to indicate kernel mode, and pushes the

old PSL and the PC of the next instruction onto the kernel stack.  The

CHMK instruction generates an exception.  The CPU goes to the System

Control Block and gets the vector to use when a CHMK exception occurs.

(The System Control Block is pointed to by EXE$GL_SCB, and the offset

into the vector table for a CHMK exception is 40 hex.)  This vector

points to routine EXE$CMODKRNL, which is where control jumps to.

#3.

Routine EXE$CMODKRNL cases the code number passed along with the CHMK

instruction to determine what routine to call.  In our case it calls

routine EXE$CMKRNL.  To increase efficiency and speed of execution, the

routine builds the call frame on the stack by hand, rather than using a

generic CALLG instruction.  The return PC address on the call frame is

specified as SRVEXIT, where an REI instruction is located.  It then

jumps to routine EXE$CMKRNL.

#4.

Routine EXE$CMKRNL calls our specified routine.  The call frame is built

up to #5.

#5.

My kernel mode routine executes with the frame pointer and stack pointer

pointing to #5.  My routine places the address of the condition handler

routine at the #5 position on the stack where it goes.

NEXT:

My routine makes a fatal mistake.  It pushes 10 arguments on the stack

and then issues a CALLS #11,SYS$FAO.  The system routine fails because

an argument is missing.  The system routine calls LIB$SIGNAL.

LIB$SIGNAL

Before LIB$SIGNAL begins its search for a condition handler, it removes

the call frame and the argument list from the stack.  The stack is

unwound down to the frame which called SYS$FAO, and the frame is removed.

Then, if bit 29 in the mask/PSW of the frame removed was set indicating

that a CALLS instruction was used, a longword containing the number of

arguments is popped from the stack, and four times the unsigned value of

the low byte of this longword is added to SP.

Everything was correct except the stack pointer now pointed one longword

below the #5 position instead of at the #5 position.  This was because

10 arguments were pushed on the stack but 11 arguments were popped off.

SEARCH FOR A CONDITION HANDLER

The signal array, signal/stop code, mechanism array, and signal argument

list are built on the stack.  A condition handler is then searched for,

frame by frame, back to the last chance exception handler.

Unfortunately the address of my condition handler was destroyed by the

signal array being built over it.

#6.

The depth argument (FFFFFFFD = -3) in the mechanism array indicates the

last chance exception handler was found.  (When analyzing a system crash

a good starting point is to look for this value near the top of the

kernel stack.  It can help you identify where the mechanism array is and

where the signal array is.)  The signal/stop code in our case also

indicates $SS_NOHANDLER.  After finding the address of a handler, the

address is placed in R1 and we JSB to SYS$CALL_HANDL.  (JSB pushes the

current PC onto the stack).

#7.

Routine SYS$CALL_HANDL issues a CALLG to the condition handler.  A frame

is built on the stack bringing us to the top of the stack.  Notice the

return PC in this top frame indicates SYS$CALL_HANDL+004.  This is the

unique identifying characteristic of a condition handler.

LAST CHANCE CONDITION HANDLER

The last chance condition handler invokes the BUG_CHECK macro which

captures total control of the CPU, copies memory to the sysdump file,

and halts the processor.

VAX/VMS 4.4 -- System Dump Analysis					25-SEP-1988 02:03:00.33#			Page 2

System crash information

Time of system crash: 25-SEP-1988 01:24:27.94#

Version of system: VAX/VMS VERSION V4.4

VAXcluster node name: HV1

Reason for BUGCHECK exception: SSRVEXCEPT, Unexpected system service exception

Process currently executing: David Deley

Current image file: HV1$DRA1:[DELEYD.WORK2]ZEROACC.EXE;138

Current IPL: 0  (decimal)

R0  = 00000000   R1  = 8000FDD2   R2  = 00000004   R3  = 7FF93EEE

R4  = 80158770   R5  = 7FFE64B4   R6  = 0000003B   R7  = 800024A8

R8  = 00000000   R9  = 800024A8   R10 = 8014080C   R11 = 00000000

AP  = 7FFE7D98   FP  = 7FFE7D80   SP  = 7FFE7D80   PC  = 8000FDD8

Current operating stack (KERNEL):

Last chance condition handler executes BUG_CHECK macro which crashes CPU.

FP,SP=> 7FFE7D80  00000000	handler

7FFE7D84  00000000	mask/PSW

7FFE7D88  00000000	AP

7FFE7D8C  7FFE7DD0	FP

7FFE7D90  80000014	PC  SYS$CALL_HANDL+004

#7.  SYS$CALL_HANDL issues CALLG to call condition handler

7FFE7D94  80017F16	PC  (JSB @#SYS$CALL_HANDL) EXE$CONTSIGNAL+07C

#6.  Execute JSB to SYS$CALL_HANDL

AP=>	7FFE7D98  00000002	signal argument list.  2 arguments follow

7FFE7D9C  7FFE7DBC	address of signal array

7FFE7DA0  7FFE7DA4	address of mechanism array

7FFE7DA4  00000004	MECHANISM ARRAY.  4 arguments follow

7FFE7DA8  7FF47560	establisher frame

7FFE7DAC  FFFFFFFD	depth (-3, last chance exception vector)

7FFE7DB0  00000001	R0 at time of exception

7FFE7DB4  00000000	R1 at time of exception

7FFE7DB8  000008F8	signal/stop code  ($SS_NOHANDLER)

7FFE7DBC  00000005	SIGNAL ARRAY.  5 arguments follow

7FFE7DC0  0000000C	exception.  value = SS$_ACCVIO

7FFE7DC4  00000000	reason mask.  read violation

7FFE7DC8  00000000	inaccessible virtual address

7FFE7DCC  000009C3	exception PC

#5.	7FFE7DD0  00000000	exception PSL	handler

7FFE7DD4  00000000			mask/psw

7FFE7DD8  7FF47578			AP  (points to user stack)

7FFE7DDC  7FFE7DE4			FP  (frame below)

7FFE7DE0  8000940C			PC  EXE$CMKRNL+00D

#4.  In EXE$CMKRNL.  It calls user specified routine.

7FFE7DE4  00000000	handler

7FFE7DE8  00000000	mask/psw

7FFE7DEC  7FF47578	AP  (arguments are on user stack)

7FFE7DF0  7FF47560	FP  (top of user stack was the previous frame)

7FFE7DF4  8000FDCE	PC  SRVEXIT

#3.  In routine EXE$CMODKRNL.  Routine calls EXE$CMKRNL.  (note AP,FP point to user stack)

From CHMK exception.

7FFE7DF8  7FFEDE96	PC  SYS$CMKRNL+006

7FFE7DFC  03C00000	old PSL

VAX/VMS 4.4 -- System Dump Analysis					25-SEP-1988 20:01:24.75#			Page 2

Process stacks

USER stack:

build	 SP =>  7FF47560  00000000	handler

call frame	7FF47564  20100000	mask/psw

of where we	7FF47568  7FF475CC	AP

were.		7FF4756C  7FF47584	FP

7FF47570  00000A57	PC  (return from $CMKRNL_S)

#2b. save reg	7FF47574  7FFE640C	save register R4

#2a. push args	7FF47578  00000002	2 arguments to SYS$CMKRNL

#2a. on stack	7FF4757C  00000800	address of our routine

#2a.		7FF47580  00000000	arglst

#2.  Program calls SYS$CMKRNL

#1.  Program starts.

7FF47584  00000000	handler  (start image)

7FF47588  00000000	mask/psw

7FF4758C  7FF475CC	AP

7FF47590  7FF475B8	FP

7FF47594  8001E448	PC  EXE$IMGSTA+02F (start image)

7FF47598  7FFE33DC

7FF4759C  00000008

7FF475A0  000009FD

7FF475A4  00000000

7FF475A8  00000000

7FF475AC  00200000

7FF475B0  7FF475CC

7FF475B4  00000000

7FF475B8  8001E449	handler  EXE$IMGSTA+030

7FF475BC  20000000	mask/psw

7FF475C0  7FF475FC	AP

7FF475C4  7FF475E8	FP

7FF475C8  7FF94218	PC

argument list.	7FF475CC  00000006

arg#1		7FF475D0  7FFE6440	MMG$IMGHDRBUF+040

arg#2		7FF475D4  7FF98BB7

arg#3		7FF475D8  7FFE640C	MMG$IMGHDRBUF+00C

arg#4		7FF475DC  7FFE64B4	MMG$IMGHDRBUF+0B4

arg#5		7FF475E0  01000028

arg#6		7FF475E4  00000000

catch all	7FF475E8  800212C0	handler  EXE$CATCH_ALL

handler	frame	7FF475EC  20000000	mask/psw

7FF475F0  00000000	AP

7FF475F4  00000000	FP       (no previous frame)

7FF475F8  7FF9417A	PC

7FF475FC  00000000

7FF47600  00000000

7FF47604  00000000

7FF47608  00000000