Documentation for the VT340 REGIS support library in DEC.FOR

START_REGIS

No arguments

If not called previously or a call to STOP_REGIS has been

made, then the VT340 REGIS mode is turned on

STOP_REGIS

no arguments

If the VT340 has had its REGIS mode turned on by a call to

START_REGIS, then this routine will turn it off

INIT_TERM

No arguments

Should be the first routine called (START_REGIS may be

called before it). Sets up the terminal and program by

doing the following:

1  CALL SMG$CREATE_VIRTUAL_KEYBOARD so keystrokes cam

be read as entered. See routine GET_CHAR

2  LUN 7 opened for output to terminal for long strings

(.gt.132 chars). Used by internal DEC.FOR library routines.

3  Downloads to the terminal the color definitions used

by DEC.FOR

Color	Color

Number	Name

14	BLACK

13	RED

12	GREEN

11	YELLOW

10	BLUE

9	WHITE

4  Erases the screen, sets the background color to green and

turns off the graphics cursor.

5  Defines the character set A1 names CARDS with the following

characters:

ASCII	CARD

A	A	(ACE)

B	2

C	3

D	4

E	5

F	6

G	7

H	8

I	9

J	10

K	J

L	Q

M	K

N	J

O	O

P	K

Q	E

R	R

a	card back pattern

b	heart

c	diamond

d	spade

e	club

f	blot (used when writing in background color

to erase)

6  Load Regis macros

A	Draw a blank card face-black border with

white background.

Cursor starts and finishes at then upper

left hand corner of card to be drawn.

B	Draw a card back-black border with pattern

defined by character 'a' in font "DECK".

Cursor starts and finishes at then upper

left hand corner of card to be drawn.

C	Erase a card from the screen.

Cursor starts and finishes at then upper

left hand corner of card to be drawn.

GET_CHAR(ICHAR)

One argument

CHARACTER*1 ICHAR

Uses SMG virtual keyboard to read one character from keyboard

without the need of a terminator. When a character is received,

it is returned without displaying it on the screen.

ASCII_SIZE(I)

INTEGER*4 FUNCTION ASCII_SIZE

One argument

INTEGER*4 I

Returns the number of characters required to display the

argument I in ASCII. Used internally by DEC library.

MOVE_CURSOR_ABS(COL,ROW)

Two arguments

INTEGER*4 COL,ROW

Moves the current position of the cursor to the absolute

position specified by COL (0-799) and ROW (0-479)

MOVE_CURSOR_REL(COL,ROW)

Two arguments

INTEGER*4 COL,ROW

Moves the position of the cursor relative to the current

position by the amounts specified in COL and ROW.

MACRO(A)

One argument

Character*1 A

Invokes the previously defined REGIS macro specified in A.

WRITE_SYMBOL(SYMBOL,COLOR)

Two arguments

CHARACTER*1 SYMBOL

INTEGER*4 COLOR

Writes the selected symbol in the selected color to the

screen. Size is 9 pixels across by 10 down. Cursor is

originally located at the upper left hand corner of the

symbol to be printed. After printing it to the screen,

the cursor has been moved to the location where the next

symbol (if any) would be placed on the line.

SYMBOL is the ASCII equivalent to the symbol in the font

DECK to be printed and COLOR is the number of the color

the symbol is to be printed in.

WRITE_LARGE_SYMBOL(SYMBOL,COLOR)

Two arguments

CHARACTER*1 SYMBOL

INTEGER*4 COLOR

Writes the selected symbol in the selected color to the

screen. Size is 9 pixels across by 20 down. Cursor is

originally located at the upper left hand corner of the

symbol to be printed. After printing it to the screen,

the cursor has been moved to the location where the next

symbol (if any) would be placed on the line.

SYMBOL is the ASCII equivalent to the symbol in the font

DECK to be printed and COLOR is the number of the color

the symbol is to be printed in.

WRITE_CHAR(SYMBOL,COLOR)

Two arguments

CHARACTER*1 SYMBOL

INTEGER*4 COLOR

Writes the selected ASCII character in the selected color

to the screen. Size is 9 pixels across by 20 down.

Cursor is originally located at the upper left hand corner of

the symbol to be printed. After printing it to the screen,

the cursor has been moved to the location where the next

symbol (if any) would be placed on the line.

SYMBOL is the ASCII character to be printed and COLOR is the

number of the color the symbol is to be printed in.

WRITE_LINE(LINE,COLOR)

Two arguments

CHARACTER*(*) LINE

INTEGER*4 COLOR

Writes the selected ASCII string in the selected color

to the screen. Character size is 9 pixels across by 20 down.

Cursor is originally located at the upper left hand corner of

the first symbol to be printed. After printing the line to the

screen,	the cursor has been moved to the location where the next

symbol (if any) would be placed on the line.

LINE is the ASCII character string to be printed and COLOR is

the number of the color the string is to be printed in.

BUILD_CARD(VALUE,SUIT,COLOR)

Three arguments

CHARACTER*1 VALUE,SUIT

INTEGER*4 COLOR

Draws a card on the screen. The cursor location at the start

and finish of this subroutine is as the upper left hand

corner of the card.

VALUE is the ASCII character equivalent of the card value

symbol in the font DECK ('A'-'M' or 'N' for joker)

SUIT is the ASCII character equivalent of the card suit

symbol in the font DECK ('b'-'e')

COLOR is the integer value specifying the color of the

card. Should be RED (13) or BLACK (14).

WAIT(SECONDS)

One argument

CHARACTER*(*) SECONDS

Pauses for the specified amount of time before returning.

Delay time is specified as a character string in VMS

delta-time format  i.e. 5 seconds would be '0 0:0:5'

CLEAR_SCREEN

No arguments

Clears the screen back to the background color.

CLEAN_UP_SCREEN

No arguments

Last call to DEC utility routines before exiting.

Clears the screen, resets the background color to its

original value, and turns off REGIS