3 Dimensional Variables

AnalytiCalc will partially support treating its worksheet as a

3 dimensional volume in which variable names can be specified as

existing on a particular "page" of the sheet by adding %nnn or

%c to the cell name. In this case, a trailing number will treat

the variable as being on page nnn. Pages start at page zero (0)

and go up. An accumulator letter can be used instead of a number

if computable page addresses are needed. Thus a valid cell might

be

B12%3

Column 2 (B) row 12 in page 3

P#2#5%2

Cell 2 columns right and 5 rows down from current cell,

in page 2

C12%D

Column 3 (C) row 12 in the page pointed at by contents

of accumulator D (so that if D contains 2, it

will be in page 2).

These pages are actually aliases for areas of the spreadsheet,

so that you get from one page to the next by adding ROWDELT to

the row number and COLDELT to the column number. (The address

wrapping mentioned in the AnalytiCalc manual takes place

subsequently.) Thus, if ROWDELT is 100 and COLDELT is 0,

you get to page 1 from page 0 by going down 100 rows.

This behavior is controlled by commands:

MDDc,r	Disables all this functionality. 3D form cell names will

not be recognized and things will look like it does not

exist. The c,r allows specification of COLDELT and ROWDELT

optionally however.

MDNc,r	Turns off 3D functionality, but 3D cell names will be

understood and treated as legal if seen. Relocated cell

names will be handled as non-3D cells and the display

will show the "normal" (huge 2D) surface. (No 3D)

This is the default startup mode.

MDEc,r	Turns 3D functionality ON (Enable) but still will treat

cell names as 2D forms when relocated.

MDFc,r	Turns on 3D functionality and FORCES it. Thus, cells

whose addresses are in pages higher than 0 will be

translated on relocation to paged forms.

When sheets are saved, unless MDD mode is turned on a record

of the current COLDELT and ROWDELT values (set in the MD commands

above and defaulting to 0 at startup) will be saved so that

these settings can be restored on spreadsheet load. If MDD

mode is on the record is neither saved nor recognized on load.

Note that MDF mode forces 3D forms in relocation so that GPR

commands (which relocate cells on load) will transform them to

3D. In MDN mode the cells would be translated to 2D forms on a

GPR load.

Ranges in depth can be used like ranges in rows or columns,

BUT the cell names must be separated by } instead of : to get

this to work. This flags that the range is a page range.

Thus a depth range would be specified by something like

C4%2}C4%6

to specify the C4 cell of pages 2 through 6 inclusive.

This is done differently from specifying simply C4%2:C4%6

because these cells are also aliased to other areas of the

sheet. In particular, suppose one has executed the command

MDF0,5

so that pages are 5 rows long and have no column offset.

Now cell C1%0 is just cell C1 (as always). But cell C1%1 is

also cell C6 (5 rows down), and cell C1%2 is cell C11 (10 rows

down), and cell C1%3 is cell C16 (15 rows down). (These are

IN THIS EXAMPLE!)

Now if one were to have a range C1%1:C1%3, and if the range

symbol were the same, this could mean either a range that

includes C1%1, C1%2, and C1%3 only, or one that includes all

the cells from C1%1 (same as C6) to C1%3 (same as C16), so

it would be equivalent to C6:C16. You are free to intermix

addressing styles in general, which causes the ambiguity.

Therefore for page ranges, you MUST use the page style

addresses and you MUST ALSO use the } symbol to indicate

that this is a page range, not a row or column range. Therefore

the page range that includes C1%1 to C1%3 is expressed

C1%1}C1%3

and is unambiguous.

Note that using aliases in page ranges will either not work

or will give bogus results. Also, page ranges must be exactly

aligned. That is, a range like C1%1}D1%3 would be illegal, and

would be treated as meaning C1%1}C1%3 instead.

It is not expected that these restrictions will be much of

a problem in actual use. The display is necessarily (with 1990's

technology, anyway) two dimensional, so some anisotropy in

handling a higher dimensional object is to be expected.  If you

choose to think of the sheet in terms of a sheaf of pages, you

will ordinarily use page addresses in ranges. These work in functions

and in copy commands.

MAPPING to Screen

You can map the pages to screen if desired in either of two

modes, allowing inspection of the pages juxtaposed as desired.

This is done with modifications of the OA and OR (Origin Absolute

and Origin Relative) commands, which remap the whole screen (OA) or

the part of the screen down and right from the cursor. The commands

are:

OAMR  or ORMR  - Map depth by rows

This command resets the mapping so that each lower row on the

screen represents a row from the next lower page of the spreadsheet.

The starting cell given in the command determines the first row's

coordinates.

The mapping resulting from a command OAMR A1 would then look

something like this:

A1	B1	C1	D1

A1%1	B1%1	C1%1	D1%1

A1%2	B1%2	C1%2	D1%2

A1%3	B1%3	C1%3	D1%3

so that each row is one page deeper in the physical sheet.

OAMC or ORMC  - Map depth by columns

This command resets the mapping so that each column further

to the right represents a column from the next lower page of the

spreadsheet. As before the starting cell determines the first row's

coordinates.

The mapping resulting from a command OAMC A1 would then look

something like this:

A1	A1%1	A1%2	A1%3

A2	A2%2	A2%2	A2%3

A3	A3%1	A3%2	A3%3

A4	A4%1	A4%2	A4%3

If these commands are followed by a D command, the system will

attempt to preserve such existing mappings as exist to the extent

possible, so that, for example, if the first column in the

OAMC A1 example above were A1,A2,B5,B6, the relative mappings

would be preserved. Pre-existing mappings in the horizontal

dimension in that case would however be lost.

It should be observed that auto screen remapping for scrolling

purposes should probably be disabled while using such mappings as these

(just use the NS command) so that they will not be totally replaced

every time your cursor ventures too far. You can define keypad commands

to move around if you like.

Glenn Everhart

3/22/1990

ADDED LATE TO V25-1B:

A VE command will enable screen repaint, undoing the effects of VM,

but will NOT repaint the screen at that command time. This is to allow

command files to switch display off and on without the tremendous overhead

of total repaints.